Star Tracker Astrometry and Star Matching

Q1)

- Problem Overview

This Assessment implements a star matching and astrometry pipeline for star centroid data obtained from space situational awareness (SSA) image.

Each centroid file contains :

- x_centroid - pixel x-coordinate of star

- y_centroid – pixel y-coordinate of star

- brightness – summed grayscale intensity around the star

This objective is to match detected stars to a star catalog (GAIA / Hipparcos) and estimate accurate RA and DEC for each detected star.

- Dataset Description

- Centroid Files

This dataset contain Three centroid files, each belonging to a different sensor :

| File Name | Sensor |
| --- | --- |
| star_centroids_1.csv | Sensor 1 |
| star_centroids_2.csv | Sensor 2 |
| star_centroids_3.csv | Sensor 3 |

Note :Astrometry is fully demonstrated for one centroid file (Sensor 1) as required.

The same pipline can be applied to the remaining files by updating sensor parameters.

- Sensor Specifications

| Sensor | Image Size (px) | Focal Length (mm) | Pixel Pitch (µm) |
| --- | --- | --- | --- |
| 1 | 6576 x 4384 | 980 | 5.5 |
| 2 | 1024 x 1024 | 4912 | 3.6 |
| 3 | 13400 x 9528 | 393 | 3.45 |

- Metadata (Initial Pointing)

| Sensor | RA (deg) | DEC (deg) | FoV_x × FoV_y (deg) |
| --- | --- | --- | --- |
| 1 | 214.958 | −47.909 | 2.03 × 1.409 |
| 2 | 101.812 | −48.565 | 0.429 × 0.429 |
| 3 | 17.283 | 13.535 | 6.732 × 4.789 |

- Software Requirements

- Python 3.8+

- Required Libraries :

[ pip install numpy pandas matplotlib ]

- Folders Attached

Digantara_Assessment

CSV/

- star_centroids_1.csv

- star_centroids_2.csv

- star_centroids_3.csv

Matched_Outputs/

- matched_star_centroids_sensor1.csv

- matched_star_centroids_sensor2.csv

- matched_star_centroids_sensor3.csv

ScreenShots Attached

star_matching.py

README.md

- Methodology

Step 1 : Load Centroid Data

- Read centroid CSV file

- Visualize star distribution

- Analyze brightness histogram

Step 2: Sensor Geometry Modeling

- Compute image center

- Compute Instantaneous Field of View (IFOV)

IFOV = pixel pitch / focal length

- Convert pixel coordinates to angular coordinates

Step 3 : Star Vector Formation

- Convert angular coordinates to unit vectors

- Normalize star direction vectors

Step 4 : Catalog Acquisition (A1)

- Download GAIA / Hipparcos catalog using RA, DEC, and FoV

- Apply magnitude filtering

- Save catalog locally

(Catalog access code is provided but commented for offline execution)

Step 5 : Pattern Creation & Star Matching (A2, A3)

- Create triangle-based angular invariant features

- Match centroid patterns with catalog patterns

- Generate initial star correspondences

Step 6 : Verification and Astrometry Solution (A4)

- Solve Wahba’s problem using matched star vectors

- Estimate rotation matrix (attitude)

- Reproject catalog stars into image frame

- Compute pixel residuals

Step 7 : Final Matched Output (A5)

- Save matched stars with :

  - Image coordinates

  - Brightness

  - RA, DEC

  - Magnitude

- Running the Code (Sensor 1 Example)

[  python star_matching.py ]

Update the Centroid file path inside the script :

[ centroid_file = r”....................\star_centroids_1.csv” ]

- Generating Results for all Three Centroid Files

To process all three sensors, update the following parameters in the script :

  - Sensor Parameters

IMAGE_WIDTH

IMAGE_HEIGHT

FOCAL_LENGTH

PIXEL_PITCH

  - Metadata Parameters

RA_center

DEC_center

FoV_x

FoV_y

  - Output File Name

    - matched_star_centroids_sensor1.csv

    - matched_star_centroids_sensor2.csv

    - matched_star_centroids_sensor3.csv

  - Output Generated

    - Centroid scatter plots

    - Brightness histograms

    - Matched star CSV files

    - Refined RA/DEC estimates

  - Notes & Assumptions

    - Reference images are used only for visual understanding

    - Brightness is used for star ranking and outlier rejection

    - The algorithm is sensor-agnostic and scalable

    - Only one centroid file is fully solved as per assignment requirement

  - Conclusion

A complete star matching and astrometry pipeline has been demonstrated using real centroid data.
The solution is robust, scalable, and directly applicable to different sensors with varying specifications.